| MAKENDI IC VE DIS TICARET LTD. Murat Reis Mah. Nuh Kuyusu Cad. Payanda Sok. No. 9, 34664 Uskudar, Istanbul / Turkiye | PRODUCT SPECIFICATION 80% DEMINERALIZED SWEET WHEY POWDER (SWP) – HIGH PROTEIN |
| --- | --- |

PRODUCT NAME & SENSORY PROPERTIES

| Product Name | 80% Demineralized Sweet Whey Powder (SWP), with High Protein |
| --- | --- |
| Product Code | MAK-80-11 |
| Product Description | Powder produced after demineralization by spray-drying method. |
| Origin | Turkey |
| Contents | Whey |
| GMO Information | Product does not contain GMO or GMO-sourced components. |
| Taste & Smell | Milk and whey specific taste and aroma; typical whey odour. |
| Appearance & Color | Very viscous, homogeneous, clean powder; light cream or white to creamy. |

PHYSICAL & CHEMICAL PROPERTIES

| Parameter | Specification | Method |
| --- | --- | --- |
| Taste | Milk and whey specific taste and aroma | TS 3707-ISO 5492 |
| Odour | Typical whey | TS 3707-ISO 5492 |
| Color | Light cream or white to creamy | TS 3707-ISO 5492 |
| Appearance | Very viscous, homogeneous, clean powder | TS 3707-ISO 5492 |
| Humidity | 1.00–5.00% | TS 1329 |
| Protein | ≥ 10% | TS 8866 |
| Lactose | 75.00–85.00% | TSE based Analysis |
| Total Minerals | Max 9.50% | ISO 2171 |
| Salt | Max 1.89% | MOHR METHOD |
| Fat (in Dry Matter) | Max 1.99% | TS EN ISO 1736 |
| Acidity (as Lactic Acid) | Max 0.20% | TS 4680 |
| pH (average of tests) | 6.00–6.50 | pH-Meter |
| Solubility (in 30 secs) | 98.0–99.9% | TS 1329 |
| Aflatoxin M1 | Negative | TS EN ISO 14501:2002 |

MICROBIOLOGICAL PROPERTIES

| Parameter | Criterion |
| --- | --- |
| Enterobacteriaceae | < 10 cfu/g ¦ ISO 21528-2 |
| Coagulase-positive Staphylococci | < 100 cfu/g ¦ ISO 6888-1 or 2 |
| Salmonella | Negative / 25 g ¦ TS 8907 ISO 6785 |
| Allergy Information | Milk, milk products and derivatives; whey itself may be allergenic. |

PACKAGING, STORAGE & SHELF LIFE

| Packaging | Net 25 kg food-grade polyethylene see-through bags with plastic-lock closure, placed in triple-layered outer packaging. |
| --- | --- |
| Storage Conditions | Store away from direct sunlight and heat, in a dry and smell-free environment at 20–25°C and max. 60% relative humidity. |
| Shelf Life | At least 12 months when stored correctly. |