| MAKENDI IC VE DIS TICARET LTD. Murat Reis Mah. Nuh Kuyusu Cad. Payanda Sok. No. 9, 34664 Uskudar, Istanbul / Turkiye | PRODUCT SPECIFICATION 90% DEMINERALIZED WHEY POWDER |
| --- | --- |

PRODUCT NAME & SENSORY PROPERTIES

| Product Name | 90% Demineralized Whey Powder |
| --- | --- |
| Product Code | MAK-90 |
| Product Description | Demineralized whey powder made from sweet cheese whey by partial demineralization (90%), concentration and spray drying into a creamy white powder. |
| Origin | Turkey |
| Contents | Whey |
| Allergen | Lactose, milk protein |
| GMO Information | Product does not contain GMO or GMO sourced components. |
| Taste & Smell | Sweet whey taste; typical whey odour. |
| Appearance & Color | Light cream or white to creamy. |

PHYSICAL & CHEMICAL PROPERTIES

| Parameter | Specification | Method |
| --- | --- | --- |
| Moisture | Max 2.00% | IDF Standard 26:1964 |
| Protein (N x 6.38) | Min 7% | AOAC International 992.15 |
| Lactose | Min 75% | AOAC 980.13 |
| Ash | Max 4.5% | TS 2131 ISO 928 |
| Titratable Acidity | Max 15% | Titration |
| pH (10% solution) | Min 6.00 | TS 11860 |
| Fat | Max 0.70% | Gerber / Teichert Method |
| Density | Min 0.600 g/cm3 | NADRG: 1978 / Volumetry Gravimetry |
| Solubility | A | ADPI |
| Scorched Particles | A | ADPI |

MICROBIOLOGICAL PROPERTIES

| Parameter | Criterion |
| --- | --- |
| Aerobic Plate Count | < 2,000 cfu/g |
| Enterobacteriaceae | Negative |
| Coagulase-positive Staphylococci | Negative |
| Yeast / Mould | Negative |
| Salmonella | Negative / 25 g |
| L. monocytogenes | Negative / 25 g |

PACKAGING, STORAGE & SHELF LIFE

| Packaging / Storage | 3-fold kraft bags with polyethylene inner liner; net 25 kg per bag. Bags should be unused, clean, waterproof, lightproof and suitable for food use. Store under low relative humidity in warehouse. Do not expose to direct sunlight or strong odours. |
| --- | --- |
| Instructions for Use | Biscuit, recombined milk, bakery, ice cream, snack foods, formulated meats, nutritional powder, chocolate, etc. |
| Shelf Life | Up to one year in unopened bag under stated storage conditions. |
| Quality Systems | ISO 9001, ISO 22000 and HACCP based quality and food-safety systems. |