| MAKENDI IC VE DIS TICARET LTD. Murat Reis Mah. Nuh Kuyusu Cad. Payanda Sok. No. 9, 34664 Uskudar, Istanbul / Turkiye | PRODUCT SPECIFICATION SCG 40 HBSO MAKENDI GLUCOSE SYRUP |
| --- | --- |

PRODUCT DESCRIPTION

| Product Name | HBSO MAKENDI GLUCOSE SYRUP |
| --- | --- |
| Product Code | SCG 40 |
| Product Description | Obtained by purified and condensed natural sugar syrup produced by hydrolisation of corn starch. |

PHYSICAL & CHEMICAL PROPERTIES

| Parameter | Unit | Min | Max | Method |
| --- | --- | --- | --- | --- |
| Brix (20 °C) | – | 84.0 | 86.0 | TL.LB.60 |
| Dry Substance | % | 81.4 | 83.3 | TL.LB.60 |
| Dextrose Equivalence (DE) | % | 37.0 | 44.0 | TL.LB.59 |
| pH | – | 4.5 | 6.5 | TL.LB.41 |
| SO₂ | mg/kg | 100 | 300 | TL.LB.29 |
| Color (ICUMSA) | IU | – | 12 | TL.LB.71 |
| Sulphated Ash | % (m/m) | – | 0.3 | TL.LB.34 |

SUGAR COMPONENTS

| Parameter | Unit | Min | Max | Method |
| --- | --- | --- | --- | --- |
| Dextrose | % | 14 | 20 | TL.LB.78 |
| Maltose | % | 12 | 17 | TL.LB.78 |

MICROBIOLOGICAL PROPERTIES

| Parameter | Unit | Max | Method |
| --- | --- | --- | --- |
| Osmophilic Yeast | CFU/g | 100 | TS 3958, Harrigan W.F. Mc Cance |
| Mold | CFU/g | 100 | TS ISO 21527-1:2008 |

HEAVY METALS

| Parameter | Unit | Max | Reference |
| --- | --- | --- | --- |
| Arsenic (As) | mg/kg | 1 | Turkish Food Codex |
| Copper (Cu) | mg/kg | 5 | Turkish Food Codex |
| Lead (Pb) | mg/kg | 0.5 | Turkish Food Codex |

GMO DECLARATION

| GMO Status | The product meets Turkish Republic Regulations No. 5977 “Biyogüvenlik Kanunu” and No. 27671 “Genetik Yapısı Değiştirilmiş Organizmalar ve Ürünlerine Dair Yönetmelik” through periodic control and analysis. Supplier information on seed, origin, storage and transportation is collected, guarantees are requested, and samples are analyzed and recorded systematically. |
| --- | --- |

PACKAGING

| Packaging | Supplied in 25 kg tin cans suitable for food preservation or in special-purpose stainless vessels as bulk. Tin-can transportation may be palletized or non-palletized according to customer request. |
| --- | --- |

STORAGE CONDITIONS & SHELF LIFE

| Storage / Shelf Life | Store indoors at room temperature (25 °C) in a dry place. Bulk product should be stored at 55–60 °C in sterilized stainless-steel vessels. Shelf life: 2 years from production date. |
| --- | --- |