| MAKENDI IC VE DIS TICARET LTD. Murat Reis Mah. Nuh Kuyusu Cad. Payanda Sok. No. 9, 34664 Uskudar, Istanbul / Turkiye | PRODUCT SPECIFICATION MAKENDI FRUCTOSE SYRUP |
| --- | --- |

PRODUCT DESCRIPTION

| Product Name | Makendi Fructose Syrup |
| --- | --- |
| Product Description | Enzyme converted natural edible, aqueous high fructose syrup produced from corn starch. |

PHYSICAL & CHEMICAL PROPERTIES

| Parameter | Unit | Min | Max | Method |
| --- | --- | --- | --- | --- |
| Brix | – | 69 | 71 | KY.LB.60 |
| Dry Substance (DS) | % | 70.4 | 72.5 | KY.LB.60 |
| pH | – | 3.5 | 5.0 | KY.LB.41 |
| SO₂ | ppm | – | 10 | KY.LB.29 |
| Color (ICUMSA) | IU | – | 20 | KY.LB.71 |
| Ash (sulfated) | % (dry basis, m/m) | – | 10 n/10g | KY.LB.60 |

SUGAR COMPONENTS

| Parameter | Unit | Min | Max | Method |
| --- | --- | --- | --- | --- |
| Fructose | % | 40 | 45 | KY.LB.78 |
| Dextrose | % | 48 | 54 | KY.LB.78 |
| Fructose + Dextrose | % | 94 | – | KY.LB.78 |

MICROBIOLOGICAL PROPERTIES

| Parameter | Unit | Max | Method |
| --- | --- | --- | --- |
| Osmophilic Yeast | CFU/g | 100 | TS 3958, Harrigan W.F. Mc Cance |
| Mold | CFU/g | 100 | TS 6580 |

HEAVY METALS

| Parameter | Unit | Max | Reference |
| --- | --- | --- | --- |
| Arsenic (As) | ppm | 1 | Turkish Food Codex |
| Copper (Cu) | ppm | 5 | Turkish Food Codex |
| Lead (Pb) | ppm | 0.5 | Turkish Food Codex |

PACKAGING

| Packaging | Bulk, or 25 kg tin cans. |
| --- | --- |

STORAGE CONDITIONS & SHELF LIFE

| Storage / Shelf Life | In tin cans, store at ambient temperature (25–30 °C). As bulk, store in stainless-steel tanks at 30–45 °C. Shelf life: 2 years. |
| --- | --- |