| MAKENDI IC VE DIS TICARET LTD. Murat Reis Mah. Nuh Kuyusu Cad. Payanda Sok. No. 9, 34664 Uskudar, Istanbul / Turkiye | PRODUCT SPECIFICATION MAKENDI GLUCOSE SYRUP S 461 |
| --- | --- |
| PRODUCT DESCRIPTION |  |  |

| Product Name | Makendi Glucose Syrup |
| --- | --- |
| Product Code | S 461 |
| Product Description | Purified and concentrated aqueous solution of nutritive saccharides obtained by hydrolysis of starch; high in polysaccharides. Transparent, water-like to light-yellow viscous liquid with a sweet taste and bland odour. |
| GMO Information | The ingredient is considered Non-GMO. Additional GM-status information is referenced in the source Product Information Sheet. |
| QUALITY & TECHNICAL SPECIFICATIONS |  |  |

| Parameter | Range / Attribute | Unit | Method |
| --- | --- | --- | --- |
| Dry substance | 80.50-81.50 | % | Refractometer (+ conversion tables) - A01 |
| Brix (20°C) | 82.90-83.90 | - | Refractometer (+ conversion tables) - A01 |
| Refractive index at 20°C | 1.4984-1.5011 | - | Refractometer (+ conversion tables) - A01 |
| DE | 40.00-43.00 | - | Lane-Eynon - A0201 (Ref. ISO 5377) |
| Dextrose on DS | 16.00-20.00 | % | HPLC - A0210 (Ref. ISO 10504) |
| Maltose on DS | 12.00-16.00 | % | HPLC - A0210 (Ref. ISO 10504) |
| SO2 | 0.00-10.00 | mg/kg | Iodine titration - A0901 |
| pH | 4.00-6.00 | - | pH - A0921 |
| Appearance of sample | OK | - | Visual - A1728 |
| Conductivity, 28 Brix | ≤150.00 | µS/cm | Conductivity meter - A1202 |
| Colour - ICUMSA | ≤15.00 | ICUMSA | Spectrophotometric - A1002 |
| Total mesophilic count | ≤1000 | n/10 g | Membrane filtration - 1922 |
| Yeasts | ≤100 | n/10 g | Membrane filtration - A1925 |
| Moulds | ≤100 | n/10 g | Membrane filtration - A1925 |
| ALLERGEN STATUS |  |  |  |  |

| Allergen Status | Absent: crustacean & shellfish; mollusk; fish; egg; milk (including lactose); peanut & oils; soybean & oils; gluten-containing wheat, rye, barley, oats, spelt, kamut and triticale; sesame & oils; celery/celeriac; mustard; lupin; tree nuts (almond, Brazil, cashew, chestnut, coconut & oils, hazelnut/filbert, hickory, macadamia, pin, pistachio, Queensland, walnut); buckwheat; bee pollen/propolis; royal jelly; pork; peach; tomato; sulfur dioxide and sulfite >10 ppm. |
| --- | --- |

NUTRITIONAL INFORMATION (PER 100 g)

| Nutrient | Value |
| --- | --- |
| Energy | 1,377 kJ |
| Calories | 324 kcal |
| Total Fat | 0 g |
| Saturated Fat | 0 g |
| Carbohydrate | 81 g |
| Sugars | 25.9 g |
| Protein | 0 g |
| Salt | 0 g |
| SENSORY PROPERTIES |  |  |

| Appearance | Color | Odor |
| --- | --- | --- |
| Syrup | Transparent water-like to light yellow | Bland |
| STORAGE, SHIPPING & SHELF LIFE |  |  |  |

| Container / Package | Storage / Handling | Shelf Life |
| --- | --- | --- |
| Bulk | 55-65°C; dry, clean area away from odorous materials; highly temperature sensitive. | 183 days |
| IBC, Drums, Plastic buckets | Store at ambient temperature; heat to max. 60°C for easy handling. | 365 days when sulfite <100 ppm; 730 days when >100 ppm |
| REGULATORY & CERTIFICATION |  |  |  |

| Regulatory Status | Complies with EU Directives and Regulations in force on foods and food ingredients. Prospective purchasers should conduct their own tests and regulatory review for intended applications. |
| --- | --- |
| Country of Manufacture | The Netherlands, Turkey |
| Halal Status | Certified Halal: Yes |
| Kosher Status | Certified Kosher: Yes; Certified Kosher for Passover: No. Certification body stated in source: London Beth Din Kashrut Division (KLBD). |
| Vegetarian Status | Vegetarian, Vegan, Ovo-Vegetarian, Lacto-Vegetarian and Lacto-Ovo-Vegetarian: Suitable |
| Organic Statement | Certified for Organic Labeling: No |