| MAKENDI IC VE DIS TICARET LTD. Murat Reis Mah. Nuh Kuyusu Cad. Payanda Sok. No. 9, 34664 Uskudar, Istanbul / Turkiye | PRODUCT SPECIFICATION WHOLE MILK POWDER |  |
| --- | --- | --- |
| PRODUCT NAME & SENSORY PROPERTIES |  |

| Product Name | Whole Milk Powder |
| --- | --- |
| Ingredients | Whole cow milk. Milk is pasteurized, condensed and spray dried. |
| Colour | White with pale creamy shade. |
| Taste & Odour | Clean, fresh pasteurized milk without foreign taste or smell. Taste of pasteurization is allowed. |
| Consistency | Fine-dispersed powder. Small lumps may be present and easily break under mechanical action. |
| Application | For use in food and industrial processing in the food industry. |

PHYSICAL & CHEMICAL PROPERTIES

| Parameter | Specification |
| --- | --- |
| Fat content | Min. 26% |
| Moisture content | Max. 5% |
| Protein in MSNF | Min. 34% |
| Purity group | Not lower than Disc B |
| Acidity | Max. 21°T |
| Solubility | Max. 0.3 cm³ |

BACTERIOLOGICAL PROPERTIES

| Parameter | Criterion |
| --- | --- |
| QMAFAnM, CFU/g | < 1 × 10⁵ |
| Coliforms | Not allowed in 0.01 g |

PACKAGING, STORAGE & SHELF LIFE

| Packaging | 25 kg kraft bags with inner polyethylene bag, sealed with a double seam. |
| --- | --- |
| Storage Conditions | Keep in a clean, well-ventilated place with relative humidity not more than 85%. |
| Shelf Life | 12 months when stored at 0–25°C. |